Intrinsic viscosity of exopolysaccharides: determination methods and functionality estimation

C Nachtigall1, C Noack1, A Furch2, L Ernst2, A M Wagemans1, D Wefers2, H Rohm1 and D Jaros1

1Chair of Food Engineering, Institute of Natural Materials Technology, Technische Universität Dresden, 01062 Dresden, Germany

2Institute of Chemistry, Food Chemistry, Martin Luther University Halle-Wittenberg, 06120 Halle (Saale), Germany

The intrinsic viscosity [η] is commonly used to describe the behaviour of hydrocolloids in aqueous solution and reflects interactions of single molecules with the respective solvent at infinite dilution. Therefore, [η] characterises the ability of a hydrocolloid to enhance viscosity, and, especially for microbial exopolysaccharides, it is known to serve as an estimation for their functionality in fermented food systems (e.g., contribution to stiffness in milk gels1). Intrinsic viscosity is affected by properties of the hydrocolloid (e.g., molecular mass, conformation, chemical structure, charge), the solvent (e.g., pH, ionic strength), and the environment (e.g., temperature). To determine [η] of hydrocolloids – solvent pairs, viscosities of a hydrocolloid concentration series are measured with capillary viscometers or rheometers and the reduced specific viscosity extrapolated to a concentration of zero. On the other hand, it is possible to determine relative viscosity directly from one single measurement with differential viscometers2. The aim of this study was to evaluate three different methods of [η] determination, using (I) concentration series and single point estimations from a rolling ball viscometer, (II) concentration-dependent flow curves from a rheometer and (III) size exclusion chromatography with coupled refractive index and viscosity detectors (SEC-RI/IV). Different commercial and fermentatively produced homo- and hetero-exopolysaccharides from lactic acid bacteria served as hydrocolloids for this study.

To allow correct calculations from SEC-RI/IV measurements, the purity and the refractive index increment dn/dc of all samples were determined. For commercial homo-exopolysaccharides, e.g. dextran (molecular mass approx. 105 Da), all three determination methods showed a good agreement of [η] (0.040 – 0.049 mL/mg). With increasing molecular mass (> 106 Da) and higher (intrinsic) viscosity, a broader variation occurred (dextran from Weissella confusa DSM20194: 0.059 – 0.156 mL/mg; hetero-exopolysaccharide from Streptococcus thermophilus DGCC7919: 0.294 – 0.343 mL/mg), presumably as a result of shear tinning effects in the capillary of the rolling ball viscometer.

In ongoing experiments, selected exopolysaccharides are analysed for [η] before and after enzymatic modification of the molecular structure (e.g., degree of branching, position of branching). Our findings contribute to the establishment of structure – functionality relationships for exopolysaccharides and allow a targeted synthesis of polymers with a defined functionality.

This work has been supported by Deutsche Forschungsgemeinschaft (DFG), project IDs: JA2033/3-1 | WE6416/4-1.

References:

1	Mende, S., Mentner, C., Thomas, S., Rohm, H., Jaros, D. (2012) Exopolysaccharide production by three different strains of Streptococcus thermophilus and its effect on physical properties of acidified milk. Engineering in Life Sciences, 12, 466-474.

2	Harding, S.E. (1997). The intrinsic viscosity of biological macromolecules. Progress in measurement, interpretation and application to structure in dilute solution. Progress in Biophysics & Molecular Biology, 68, 207–262.